����                          ����

�                              �   �

������  ���  �   � ����  ����  ������ �      ���  ����� ������ ����� ����

� �   � �   � �   � �   � �    � �     ����� �����   �    ���   �����

� �   � �   � �   � �   � �    � �     �   �     �   �    �     �   �

������ �����  ���� �   � ����  ������ ����� �   � �����   �    ����� �   �

Programming Information v0.90

January 29, 1995

Andr� Baresel - Craig Jackson

Copyright (c) 1995

Removed all references to SBlaster Only Ports.

==============================================================================

SOUNDBLASTER MODELS

------------------------------------------------------------------------------

SoundBlaster 1.0

SoundBlaster 1.5

SoundBlaster 2.0

SoundBlaster MCV (MCA)

SoundBlaster Pro

SoundBlaster Pro2

SoundBlaster Pro Value Edition

SoundBlaster Pro MCV (MCA)

SoundBlaster 16

SoundBlaster 16 Basic Edition

SoundBlaster 16 Value Edition

SoundBlaster 16 ASP

SoundBlaster 16 MultiCD

SoundBlaster 16 MultiCD ASP

SoundBlaster 16 SCSI-2

SoundBlaster 16 SCSI-2 ASP

SoundBlaster AWE32

SoundBlaster AWE32 Value Edition

���������������������������������������������������������������ͻ

� SOUNDBLASTER MODEL    �  8-BIT Hz (DAC)   �  16-BIT Hz (DAC)  �

�                       �       MONO        �       MONO        �

���������������������������������������������������������������͹

�  SoundBlaster 1.x     �    4000-22222     �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster 2.x     �    4000-45454     �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster Pro     �    4000-45454     �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster 16      �    4000-45454     �     4000-45454    �

���������������������������������������������������������������Ķ

�  SoundBlaster AWE32   �    5000-45454     �     5000-45454    �

���������������������������������������������������������������ͼ

���������������������������������������������������������������ͻ

� SOUNDBLASTER MODEL    �  8-BIT Hz (DAC)   �  16-BIT Hz (DAC)  �

�                       �      STEREO       �      STEREO       �

���������������������������������������������������������������͹

�  SoundBlaster 1.x     �        N/A        �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster 2.x     �        N/A        �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster Pro     �    4000-22727     �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster 16      �    4000-45454     �     4000-45454    �

���������������������������������������������������������������Ķ

�  SoundBlaster AWE32   �    5000-45454     �     5000-45454    �

���������������������������������������������������������������ͼ

���������������������������������������������������������������ͻ

� SOUNDBLASTER MODEL    �  8-BIT Hz (ADC)   �  16-BIT Hz (ADC)  �

�                       �       MONO        �       MONO        �

���������������������������������������������������������������͹

�  SoundBlaster 1.x     �    4000-11111     �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster 2.x     �    4000-15151     �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster Pro     �    4000-45454     �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster 16      �    4000-45454     �     4000-45454    �

���������������������������������������������������������������Ķ

�  SoundBlaster AWE32   �    5000-45454     �     5000-45454    �

���������������������������������������������������������������ͼ

���������������������������������������������������������������ͻ

� SOUNDBLASTER MODEL    �  8-BIT Hz (ADC)   �  16-BIT Hz (ADC)  �

�                       �      STEREO       �      STEREO       �

���������������������������������������������������������������͹

�  SoundBlaster 1.x     �        N/A        �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster 2.x     �        N/A        �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster Pro     �    4000-22727     �        N/A        �

���������������������������������������������������������������Ķ

�  SoundBlaster 16      �    4000-45454     �     4000-45454    �

���������������������������������������������������������������Ķ

�  SoundBlaster AWE32   �    5000-45454     �     5000-45454    �

���������������������������������������������������������������ͼ

==============================================================================

SOUNDBLASTER COMPATIBLES

------------------------------------------------------------------------------

???                         AudioDrive 688

???                         Laserwave Supra 16

???                         MediaConcept Pro

Adaptec                     Audio Machine???

Addonics???                 Sound Master

Add Tech                    Sound 2000

Advanced Gravis             Gravis Ultrasound

Gravis Ultrasound MAX

Alpha Systems               VR

ATI                         ATI Stereo F/X

AVM Technology              AltraPro

Aztech                      Sound Galaxy???

Sound Galaxy BXII

Sound Galaxy BXII Extra

Sound Galaxy Basic 16

Sound Galaxy Basic 16 Extra

Sound Galaxy Nova 16

Sound Galaxy NXII

Sound Galaxy NX Pro

Sound Galaxy Orion 16

Sound Galaxy Pro???

Sound Galaxy Pro 16

Biostar???                  MediaChips Audio Device 16 (7168VMD)

Bluepoint                   Sound FX 3000???

Cardinal Technologies       Digital Sound Pro 16

Sound Pilot

Computer Peripherals        ViVa Maestro 16

ViVa Maestro 16 VR

CPS                         AudioBlaster 1.0

AudioBlaster 1.5

AudioBlaster 2.0

AudioBlaster 2.5

AudioBlaster 4.0

Diamond                     Sonic Sound

Sonic Sound LX

DSP Solutions               PORT*ABLE Sound Plus

Ensoniq                     Ensoniq Soundscape

Gallant                     Audio Plus True 16

Genoa Systems               AudioBahn 16 Pro

IBM                         IBM Windsurfer

Kingston Technologies       Omnivox

Logitech Inc.               Soundman 16

Soundman Wave

Lyben                       Sound Card Basic

Sound Card 16 Stereo

MediaMagic                  OnMagic???

MediaTrix                   AudioTrix Pro

MediaVision                 Deluxe

Premium 3D

Premium 3D MultiCD

Premium 3D SCSI-2

Pro 3D

Pro Audio 16 Basic

Pro Audio Spectrum 16

Pro Audio Studio

Pro Sonic 16

MEI                         Premium 16 Sound Card

Microsoft                   Windows Sound System 2.0???

MultiWave Innovation Inc.   AudioWave Platinum 16

Nu Reality                  Vivid 3D???

Oak Technologies            Mozart-128

Orchid                      GameWave 32

Sound Producer

SoundWave 32

Paradise                    Audio Basic

Audio Professional

ProLink Computer Inc.       SoundPlus

Prometheus                  Aria 16

Aria 16se

Reveal                      Sound FX/32 Wavetable (SC600)

Sigma Designs               Reel Magic

Triumph Logistic Computers  Audio Forge Professional

Trust                       Sound Expert 16 Deluxe

Turtle Beach                Monte Carlo

Tropez

Yamaha                      Yamaha CBX-B1

Zoltrix                     TESS

==============================================================================

ENVIRONMENT VARIABLE

------------------------------------------------------------------------------

BLASTER=Aa Ii Dd Hh Pp Tt

Aa - Base Address                    0210h, 0220h, 0230h, 0240h,

0250h, 0260h, 0280h

Ii - Interrupt Request                     2, 3, 5, 7, 10

Dd - DMA Channel, 8-bit                       0, 1, 3

Hh - DMA Channel, 16-bit                      5, 6, 7

Pp - Base MIDI Address                      0300h, 0330h

Tt - Model                        1(1.x), 2(Pro), 3(2.0), 4(Pro2.0),

5(ProMCV), 6(16, AWE32)

NOTE: 16-bit DMA definition may actually refer to an 8-bit DMA alias.

==============================================================================

HARDWARE PORTS

------------------------------------------------------------------------------

02x00h         FM Music - Left Status Port             Read        SBPro

02x00h         FM Music - Left Register Port           Write       SBPro

02x01h         FM Music - Left Data Register           Write       SBPro

02x02h         FM Music - Right Status Port            Read        SBPro

02x02h         FM Music - Right Register Port          Write       SBPro

02x03h         FM Music - Right Data Register          Write       SBPro

02x04h         Mixer - Register Port                   Write       SBPro

02x05h         Mixer - Data Register                   Read/Write  SBPro

02x06h         DSP - Reset                             Write       SB

02x08h         FM Music - Compatible Status Port       Read        SB

02x08h         FM Music - Compatible Register Port     Write       SB

02x09h         FM Music - Compatible Data Register     Write       SB

02x0Ah         DSP - Read Data                         Read        SB

02x0Ch         DSP - Write Data or Command             Write       SB

02x0Ch         DSP - Write Buffer Status               Read        SB

02x0Eh         DSP - Data Available Status             Read        SB

02x0Eh         DSP - IRQ Acknowledge, 8-bit            Read        SB

02x10h         CD-ROM - Data Register                  Read        SBPro

02x10h         CD-ROM - Command Port                   Write       SBPro

02x11h         CD-ROM - Status Port                    Read        SBPro

02x12h         CD-ROM - Reset                          Write       SBPro

02x13h         CD-ROM - Enable                         Write       SBPro

0388h          AdLib - Status Port                     Read        SB

0388h          AdLib - Register Port                   Write       SB

0389h          AdLib - Data Register                   Write       SB

0200h-0207h    Joystick                                Varies      SB

------------------------------------------------------------------------------

02x00h     FM Music - Left Status Port                 Read        SBPro

DESCRIPTION

Categorizes left-channel synthesizer generated timer interrupts.

STATUS BYTE

�������������������������������ͻ

� 7 � 6 � 5 �[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (1)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �������������������

�   �   ������������������������ Timer 2 Status      (1 = Expired)

�   ���������������������������� Timer 1 Status      (1 = Expired)

�������������������������������� Global Timer Status (1 = Active )

NOTES

� Global Timer Status will be active if either timer has expired.

� Counter period of Timer 1 is 80�s, and Timer 2 is 230�s.

� Synthesizer generated timer interrupts are signalled on IRQ0.

SEE ALSO

02x00h  FM Music - Left Register Port      Write

02x01h  FM Music - Left Data Register      Write

�����������������������������������������������������

02x08h  FM Music - Compat. Status Port     Read

0338h  AdLib    - Status Port             Read

�����������������������������������������������������

02x02h  FM Music - Right Status Port       Read

------------------------------------------------------------------------------

02x00h     FM Music - Left Register Port               Write       SBPro

DESCRIPTION

Selects register index into left-channel synthesizer data port (02x01h)

PROCEDURE

a) Write register index (02x00h)

b) Wait 3.3�s (0.0?�s OPL3)

c) Write register data  (02x01h)

d) Wait 23�s  (0.28�s OPL3)

NOTES

� Original SoundBlaster Pros were equipped with dual OPL2 synthesizers,

connecting this channel to the left speaker exclusively; whereas later

models began using OPL3-compatible synthesizers, which send all output

to both the left and right speakers unless programmed to do otherwise.

SEE ALSO

02x00h  FM Music - Left Status Port        Read

02x01h  FM Music - Left Data Register      Write

�����������������������������������������������������

02x08h  FM Music - Compat. Register Port   Write

0338h  AdLib    - Register Port           Write

�����������������������������������������������������

02x02h  FM Music - Right Register Port     Write

------------------------------------------------------------------------------

02x01h     FM Music - Left Data Register               Write       SBPro

DESCRIPTION

References currently selected left-channel synthesizer index register.

PROCEDURE

a) Write register index (02x00h)

b) Wait 3.3�s (0.0?�s OPL3)

c) Write register data  (02x01h)

d) Wait 23�s  (0.28�s OPL3)

NOTES

� Original SoundBlaster Pros were equipped with dual OPL2 synthesizers,

connecting this channel to the left speaker exclusively; whereas later

models began using OPL3-compatible synthesizers, which send all output

to both the left and right speakers unless programmed to do otherwise.

SEE ALSO

02x00h  FM Music - Left Status Port        Read

02x00h  FM Music - Left Register Port      Write

�����������������������������������������������������

02x09h  FM Music - Compat. Data Register   Write

0339h  AdLib    - Data Register           Write

�����������������������������������������������������

02x03h  FM Music - Right Data Register     Write

------------------------------------------------------------------------------

02x02h     FM Music - Right Status Port                Read        SBPro

DESCRIPTION

Categorizes right-channel synthesizer generated timer interrupts.

STATUS BYTE

�������������������������������ͻ

� 7 � 6 � 5 �[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (1)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �������������������

�   �   ������������������������ Timer 2 Status      (1 = Expired)

�   ���������������������������� Timer 1 Status      (1 = Expired)

�������������������������������� Global Timer Status (1 = Active )

NOTES

� Global Timer Status will be active if either timer has expired.

� Counter period of Timer 1 is 80�s, and Timer 2 is 230�s.

� Synthesizer generated timer interrupts are signalled on IRQ0.

SEE ALSO

02x02h  FM Music - Right Register Port     Write

02x03h  FM Music - Right Data Register     Write

�����������������������������������������������������

038Ah  A. Adlib - Status Port             Read

�����������������������������������������������������

02x00h  FM Music - Left Status Port        Read

------------------------------------------------------------------------------

02x02h     FM Music - Right Register Port              Write       SBPro

DESCRIPTION

Selects register index into right-channel synthesizer data port (02x03h)

PROCEDURE

a) Write register index (02x02h)

b) Wait 3.3�s (0.0?�s OPL3)

c) Write register data  (02x03h)

d) Wait 23�s  (0.28�s OPL3)

NOTES

� Original SoundBlaster Pros were equipped with dual OPL2 synthesizers,

connecting this channel to the right speaker exclusively; whereas later

models began using OPL3-compatible synthesizers, which send all output

to both the left and right speakers unless programmed to do otherwise.

SEE ALSO

02x02h  FM Music - Right Status Port       Read

02x03h  FM Music - Right Data Register     Write

�����������������������������������������������������

038Ah  A. Adlib - Register Port           Write

�����������������������������������������������������

02x00h  FM Music - Left Register Port      Write

------------------------------------------------------------------------------

02x03h     FM Music - Right Data Register              Write       SBPro

DESCRIPTION

References currently selected right-channel synthesizer index register.

PROCEDURE

a) Write register index (02x02h)

b) Wait 3.3�s (0.0?�s OPL3)

c) Write register data  (02x03h)

d) Wait 23�s  (0.28�s OPL3)

NOTES

� Original SoundBlaster Pros were equipped with dual OPL2 synthesizers,

connecting this channel to the right speaker exclusively; whereas later

models began using OPL3-compatible synthesizers, which send all output

to both the left and right speakers unless programmed to do otherwise.

SEE ALSO

02x02h  FM Music - Right Status Port       Read

02x02h  FM Music - Right Register Port     Write

�����������������������������������������������������

038Bh  A. Adlib - Data Register           Write

�����������������������������������������������������

02x01h  FM Music - Left Data Register      Write

------------------------------------------------------------------------------

02x04h     Mixer - Register Port                       Write       SBPro

DESCRIPTION

Selects register index into mixer data port (02x05h)

SELECTION BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Mixer Register Index bit   0 (SB16)

�   �   �   �   �   �   �������� Mixer Register Index bit 3-1 (SBPro)

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Mixer Register Index bit   4 (SB16)

�   �   ������������������������ Mixer Register Index bit 7-5 (SBPro)

�   �������������������������Ĵ

�������������������������������

PROCEDURE

a) Write mixer index     (02x04h)

b) Read/write mixer data (02x05h)

NOTES

� See complete mixer reference later in this document.

� MediaVision 3D-series cards use this port for index and data output.

SEE ALSO

02x05h  Mixer - Data Register              Read/Write

------------------------------------------------------------------------------

02x05h     Mixer - Data Register                       Read/Write  SBPro

DESCRIPTION

References currently selected mixer index register.

PROCEDURE

a) Write mixer index     (02x04h)

b) Read/write mixer data (02x05h)

NOTES

� See complete mixer reference later in this document.

SEE ALSO

02x05h  Mixer - Register Port              Write

------------------------------------------------------------------------------

02x06h     DSP - Reset                                 Write       SB

DESCRIPTION

Performs complete reset of DSP, terminating all pending operations.

PROCEDURE

a) Write 001h

b) Wait 3.3�s minimum

c) Write 000h

d) Wait 100�s maximum for DSP Data Available (02x0Eh)

e) Read 0AAh from DSP Read Data (02x0Ah)

NOTES

� Reset of DSP disables speaker (see DSP command 0D3h).

------------------------------------------------------------------------------

02x08h     FM Music - Compatible Status Port           Read        SB

DESCRIPTION

Categorizes left-channel synthesizer generated timer interrupts.

STATUS BYTE

�������������������������������ͻ

� 7 � 6 � 5 �[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (1)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �������������������

�   �   ������������������������ Timer 2 Status      (1 = Expired)

�   ���������������������������� Timer 1 Status      (1 = Expired)

�������������������������������� Global Timer Status (1 = Active )

NOTES

� Global Timer Status will be active if either timer has expired.

� Counter period of Timer 1 is 80�s, and Timer 2 is 230�s.

� Synthesizer generated timer interrupts are signalled on IRQ0.

� Alias for FM Music - Left Status Port (02x00h) on SBPro or higher.

SEE ALSO

02x08h  FM Music - Compat. Register Port   Write

02x09h  FM Music - Compat. Data Register   Write

�����������������������������������������������������

02x00h  FM Music - Left Status Port        Read

------------------------------------------------------------------------------

02x08h     FM Music - Compatible Register Port         Write       SB

DESCRIPTION

Selects register index into left-channel synthesizer data port (02x09h)

PROCEDURE

a) Write register index (02x08h)

b) Wait 3.3�s (0.0?�s OPL3)

c) Write register data  (02x09h)

d) Wait 23�s  (0.28�s OPL3)

NOTES

� Original SoundBlaster Pros were equipped with dual OPL2 synthesizers,

connecting this channel to the left speaker exclusively; whereas later

models began using OPL3-compatible synthesizers, which send all output

to both the left and right speakers unless programmed to do otherwise.

� Alias for FM Music - Left Register Port (02x00h) on SBPro or higher.

SEE ALSO

02x08h  FM Music - Compat. Status Port     Read

02x09h  FM Music - Compat. Data Register   Write

�����������������������������������������������������

02x00h  FM Music - Left Register Port      Write

------------------------------------------------------------------------------

02x09h     FM Music - Compatible Data Register         Write       SB

DESCRIPTION

References currently selected left-channel synthesizer index register.

PROCEDURE

a) Write register index (02x08h)

b) Wait 3.3�s (0.0?�s OPL3)

c) Write register data  (02x09h)

d) Wait 23�s  (0.28�s OPL3)

NOTES

� Original SoundBlaster Pros were equipped with dual OPL2 synthesizers,

connecting this channel to the left speaker exclusively; whereas later

models began using OPL3-compatible synthesizers, which send all output

to both the left and right speakers unless programmed to do otherwise.

� Alias for FM Music - Left Data Register (02x01h) on SBPro or higher.

SEE ALSO

02x08h  FM Music - Compat. Status Port     Read

02x08h  FM Music - Compat. Register Port   Write

�����������������������������������������������������

02x01h  FM Music - Left Data Register      Write

------------------------------------------------------------------------------

02x0Ah     DSP - Read Data                             Read        SB

DESCRIPTION

Input port for DSP data byte reads.

PROCEDURE

a) Loop until bit 7 = 1 of DSP Data Available Status (02x0Eh read)

b) Input byte from DSP Read Data port (02x0Ah read)

NOTES

� Hard system resets occasionally cause some DSPs to leave an

extraneous data byte on the bus (typically 0AAh).  Perform

an explicit DSP reset at application startup to correct for this.

SEE ALSO

02x0Eh  DSP - Data Available Status        Read

�����������������������������������������������������

02x0Ch  DSP - Write Data or Command        Write

�����������������������������������������������������

02x06h  DSP - Reset                        Write

----------------------------------------------------------------------------

02x0Ch     DSP - Write Data or Command                 Write       SB

DESCRIPTION

Output port for DSP command and data byte writes.

PROCEDURE

a) Loop until bit 7 = 0 of DSP Write Buffer Status (02x0Ch read)

b) Output byte to DSP Write Data or Command port (02x0Ch write)

SEE ALSO

02x0Ch  DSP - Write Buffer Status          Read

�����������������������������������������������������

02x0Ah  DSP - Read Data                    Read

------------------------------------------------------------------------------

02x0Ch     DSP - Write Buffer Status                   Read        SB

DESCRIPTION

Indicates whether or not DSP is ready to receive data through

the DSP Write Data or Command port (02x0Ch write).

STATUS BYTE

�������������������������������ͻ

� 7 �[6]�[5]�[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (?)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   ���������������������������

�������������������������������� Write Buffer Status (0 = Ready)

PROCEDURE

a) Loop until bit 7 = 0 of DSP Write Buffer Status (02x0Ch read)

b) Output byte to DSP Write Data or Command port (02x0Ch write)

SEE ALSO

02x0Ch  DSP - Write Data or Command        Write

�����������������������������������������������������

02x0Eh  DSP - Data Available Status        Read

------------------------------------------------------------------------------

02x0Eh     DSP - Data Available Status                 Read        SB

DESCRIPTION

Indicates whether or not DSP has pending data to be read through

the DSP Read Data port (02x0Ah read).

STATUS BYTE

�������������������������������ͻ

� 7 �[6]�[5]�[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (?)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   ���������������������������

�������������������������������� Read Status (1 = Ready)

PROCEDURE

a) Loop until bit 7 = 1 of DSP Data Available Status (02x0Eh read)

b) Input byte from DSP Read Data port (02x0Ah read)

SEE ALSO

02x0Ah  DSP - Read Data                    Read

�����������������������������������������������������

02x0Ch  DSP - Write Buffer Status          Read

------------------------------------------------------------------------------

02x0Eh     DSP - IRQ Acknowledge, 8-bit                Read        SB

DESCRIPTION

Acknowledges an IRQ signalled by an 8-bit DSP operation.

PROCEDURE

a) IRQ: Read from IRQ Acknowledge, 8-bit port (02x0Eh)

b) IRQ: Perform Generic EOI (020h) to appropriate PICs

NOTES

� Reflects correct DSP Data Available Status from IRQ.

SEE ALSO

02x0Fh  DSP - IRQ Acknowledge, 16-bit      Read

------------------------------------------------------------------------------

02x10h     CD-ROM - Data Register                      Read        SBPro

DESCRIPTION

Input port for active CD-ROM drive data and command response reads.

PROCEDURE

a) Loop until bit 2 = 0 or bit 1 = 0 of CD-ROM Status Port (02x11h read)

b) Input byte from CD-ROM Data Register port (02x10h read)

SEE ALSO

02x11h  CD-ROM - Status Port               Read

�����������������������������������������������������

02x13h  CD-ROM - Enable                    Write

�����������������������������������������������������

02x10h  CD-ROM - Command Port              Write

------------------------------------------------------------------------------

02x10h     CD-ROM - Command Port                       Write       SBPro

DESCRIPTION

Output port for active CD-ROM drive command sequences.

PROCEDURE

???

NOTES

� Command packets are seven bytes long.

� Interrupts should be disabled during output.

SEE ALSO

02x13h  CD-ROM - Enable                    Write

�����������������������������������������������������

02x10h  CD-ROM - Data Register             Read

------------------------------------------------------------------------------

02x11h     CD-ROM - Status Port                        Read        SBPro

DESCRIPTION

Indicates whether or not active CD-ROM drive is waiting for command data,

has pending data available, or has prepared a command response.

STATUS BYTE

�������������������������������ͻ

�[7]�[6]�[5]�[4]�[3]� 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Write Status   (0 = Busy, v1.0+)

�   �   �   �   �   �   �������� Data Available (0 = Ready)

�   �   �   �   �   ������������ Read Status    (0 = Ready)

�   �   �   �   ���������������� Reserved (1)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

SEE ALSO

02x10h  CD-ROM - Data Register             Read

02x10h  CD-ROM - Command Port              Write

�����������������������������������������������������

02x13h  CD-ROM - Enable                    Write

------------------------------------------------------------------------------

02x12h     CD-ROM - Reset                              Write       SBPro

DESCRIPTION

Performs complete reset of active CD-ROM drive.

PROCEDURE

a) Write 001h

b) Wait 82�s??? minimum

c) Write 000h

NOTES

� Reset will close open drive doors.

SEE ALSO

02x13h  CD-ROM - Enable                    Write

------------------------------------------------------------------------------

02x13h     CD-ROM - Enable                             Write       SBPro

DESCRIPTION

Selects active CD-ROM drive on multiple drive systems.

PROCEDURE

???

SEE ALSO

02x10h  CD-ROM - Data Register             Read

02x10h  CD-ROM - Command Port              Write

02x11h  CD-ROM - Status Port               Read

02x12h  CD-ROM - Reset                     Write

------------------------------------------------------------------------------

0388h      AdLib - Status Port                         Read        SB

DESCRIPTION

Categorizes left-channel synthesizer generated timer interrupts.

STATUS BYTE

�������������������������������ͻ

� 7 � 6 � 5 �[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (1)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �������������������

�   �   ������������������������ Timer 2 Status      (1 = Expired)

�   ���������������������������� Timer 1 Status      (1 = Expired)

�������������������������������� Global Timer Status (1 = Active )

NOTES

� Global Timer Status will be active if either timer has expired.

� Counter period of Timer 1 is 80�s, and Timer 2 is 230�s.

� Synthesizer generated timer interrupts are signalled on IRQ0.

� Alias for FM Music - Left Status Port (02x00h).

SEE ALSO

0388h   AdLib - Register Port              Write

0389h   AdLib - Data Register              Write

�����������������������������������������������������

02x00h  FM Music - Left Status Port        Read

------------------------------------------------------------------------------

0388h      AdLib - Register Port                       Write       SB

DESCRIPTION

Selects register index into left-channel synthesizer data port (0389h)

PROCEDURE

a) Write register index (0388h)

b) Wait 3.3�s (0.0?�s OPL3)

c) Write register data  (0389h)

d) Wait 23�s  (0.28�s OPL3)

NOTES

� Original SoundBlaster Pros were equipped with dual OPL2 synthesizers,

connecting this channel to the left speaker exclusively; whereas later

models began using OPL3-compatible synthesizers, which send all output

to both the left and right speakers unless programmed to do otherwise.

� Alias for FM Music - Left Register Port (02x00h).

SEE ALSO

0388h   AdLib - Status Port                Read

0389h   AdLib - Data Register              Write

�����������������������������������������������������

02x00h  FM Music - Left Register Port      Write

------------------------------------------------------------------------------

0389h      AdLib - Data Register                       Write       SB

DESCRIPTION

References currently selected left-channel synthesizer index register.

PROCEDURE

a) Write register index (0388h)

b) Wait 3.3�s (0.0?�s OPL3)

c) Write register data  (0389h)

d) Wait 23�s  (0.28�s OPL3)

NOTES

� Original SoundBlaster Pros were equipped with dual OPL2 synthesizers,

connecting this channel to the left speaker exclusively; whereas later

models began using OPL3-compatible synthesizers, which send all output

to both the left and right speakers unless programmed to do otherwise.

� Alias for FM Music - Left Data Register (02x01h).

SEE ALSO

0388h   AdLib - Status Port                Read

0388h   AdLib - Register Port              Write

�����������������������������������������������������

02x01h  FM Music - Left Data Register      Write

------------------------------------------------------------------------------

0201h      Joystick Status Port                        Read/Write  SB

DESCRIPTION

Status port for joystick button and position read.

JOYSTICK BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Joystick A: Switch X (1 = Reached)

�   �   �   �   �   �   �������� Joystick A: Switch Y (1 = Reached)

�   �   �   �   �   ������������ Joystick B: Switch X (1 = Reached)

�   �   �   �   ���������������� Joystick B: Switch Y (1 = Reached)

�   �   �   �������������������� Joystick A: Button 1 (0 = Active )

�   �   ������������������������ Joystick A: Button 2 (0 = Active )

�   ���������������������������� Joystick B: Button 1 (0 = Active )

�������������������������������� Joystick B: Button 2 (0 = Active )

PROCEDURE

a) Output byte to Joystick Status Port (0201h read/write) to reset timer

b) Loop until all directional bits charge in Joystick Status Port (0201h

read/write), use elapsed time waiting on each bit proportionally

==============================================================================

DSP COMMANDS

------------------------------------------------------------------------------

010h           Direct DAC, 8-bit                                   SB

014h           DMA DAC, 8-bit                                      SB

016h           DMA DAC, 2-bit ADPCM                                SB

017h           DMA DAC, 2-bit ADPCM Reference                      SB

01Ch           Auto-Initialize DMA DAC, 8-bit                      SB2.0

01Fh           Auto-Initialize DMA DAC, 2-bit ADPCM Reference      SB2.0

020h           Direct ADC, 8-bit                                   SB

024h           DMA ADC, 8-bit                                      SB

02Ch           Auto-Initialize DMA ADC, 8-bit                      SB2.0

030h           MIDI Read Poll                                      SB

031h           MIDI Read Interrupt                                 SB

034h           MIDI Read Poll + Write Poll (UART)                  SB2.0

038h           MIDI Write Poll                                     SB

040h           Set Time Constant                                   SB

048h           Set DMA Block Size                                  SB2.0

074h           DMA DAC, 4-bit ADPCM                                SB

075h           DMA DAC, 4-bit ADPCM Reference                      SB

076h           DMA DAC, 2.6-bit ADPCM                              SB

077h           DMA DAC, 2.6-bit ADPCM Reference                    SB

07Dh           Auto-Initialize DMA DAC, 4-bit ADPCM Reference      SB2.0

07Fh           Auto-Initialize DMA DAC, 2.6-bit ADPCM Reference    SB2.0

080h           Silence DAC                                         SB

0D0h           Halt DMA Operation, 8-bit                           SB

0D1h           Enable Speaker                                      SB

0D3h           Disable Speaker                                     SB

0D4h           Continue DMA Operation, 8-bit                       SB

0D8h           Speaker Status                                      SB

0DAh           Exit Auto-Initialize DMA Operation, 8-bit           SB2.0

0E0h           DSP Identification                                  SB2.0

0E1h           DSP Version                                         SB

0E4h           Write Test Register                                 SB2.0

0E8h           Read Test Register                                  SB2.0

0F0h           Sine Generator                                      SB

0F2h           IRQ Request, 8-bit                                  SB

------------------------------------------------------------------------------

010h       Direct DAC, 8-bit                                       SB

COMMAND->SAMPLEBYTE

DESCRIPTION

Outputs single sample.

PROCEDURE

a) Send Direct DAC, 8-bit command (010h) and sample

b) Wait for correct timing

NOTES

� Direct mode maximum sample rate is 23KHz.

SEE ALSO

020h   Direct ADC, 8-bit

------------------------------------------------------------------------------

014h       DMA DAC, 8-bit                                          SB

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Initiates 8-bit DMA transfer.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Enable Speaker command (0D1h)

d) Setup DMA controller (mode = 048h + channel)

e) Perform DMA DAC, 8-bit command (014h)

f) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

g) Perform Disable Speaker command (0D3h)

LENGTH = SAMPLES - 1

NOTES

� Use command 0Cxh to avoid SoundBlaster 16 quantization errors.

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

0D4h   Continue DMA Operation, 8-bit

�����������������������������������������������������

01Ch   Auto-Initialize DMA DAC, 8-bit

090h   Auto-Initialize DMA DAC, 8-bit (High Speed)

0Cxh   Generic DAC/ADC DMA, 8-bit

�����������������������������������������������������

017h   DMA DAC, 2-bit ADPCM Reference

075h   DMA DAC, 4-bit ADPCM Reference

077h   DMA DAC, 2.6-bit ADPCM Reference

�����������������������������������������������������

024h   DMA ADC, 8-bit

------------------------------------------------------------------------------

016h       DMA DAC, 2-bit ADPCM                                    SB

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Initiates 2-bit ADPCM DMA transfer with accumulated reference byte.

This operation uses 8-bit DMA mode.

PROCEDURE (after command 017h)

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Enable Speaker command (0D1h)

d) Setup DMA controller (mode = 048h + channel)

e) Perform DMA DAC, 2-bit ADPCM command (016h)

f) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

g) Perform Disable Speaker command (0D3h)

LENGTH = (SAMPLES-1 + 3)/4

NOTES

� Supports up to 11KHz on SoundBlaster 1.x

� Ensoniq Soundscape does not support ADPCM.

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

0D4h   Continue DMA Operation, 8-bit

�����������������������������������������������������

017h   DMA DAC, 2-bit ADPCM Reference

�����������������������������������������������������

074h   DMA DAC, 4-bit ADPCM

076h   DMA DAC, 2.6-bit ADPCM

------------------------------------------------------------------------------

017h       DMA DAC, 2-bit ADPCM Reference                          SB

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Initiates 2-bit ADPCM DMA transfer with new reference byte.  This

operation uses 8-bit DMA mode.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Enable Speaker command (0D1h)

d) Setup DMA controller (mode = 048h + channel)

e) Perform DMA DAC, 2-bit ADPCM Reference command (017h)

f) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

g) Perform Disable Speaker command (0D3h)

LENGTH = (SAMPLES-1 + 3)/4 + 1

NOTES

� Supports up to 11KHz on SoundBlaster 1.x

� Ensoniq Soundscape does not support ADPCM.

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

0D4h   Continue DMA Operation, 8-bit

�����������������������������������������������������

016h   DMA DAC, 2-bit ADPCM

�����������������������������������������������������

075h   DMA DAC, 4-bit ADPCM Reference

077h   DMA DAC, 2.6-bit ADPCM Reference

------------------------------------------------------------------------------

01Ch       Auto-Initialize DMA DAC, 8-bit                          SB2.0

COMMAND

DESCRIPTION

Initiates auto-initialize 8-bit DMA transfer.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Set DMA Block Size command (048h)

d) Perform Enable Speaker command (0D1h)

e) Setup DMA controller (mode = 058h + channel)

f) Perform Auto-Initialize DMA DAC, 8-bit command (01Ch)

g) IRQ: Prepare next half of buffer (not always in handler)

h) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

i) Loop to G until complete

j) Perform Disable Speaker command (0D3h)

k) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

l) Perform Exit Auto-Initialize DMA Operation, 8-bit command (0DAh)

m) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

NOTES

� Supports up to 23KHz (11.5KHz stereo???)

� Exit auto-initialized mode by programming single-cycle DMA output or

with Exit Auto-Initialize DMA Operation, 8-bit (0DAh).

� Use command 0Cxh to avoid SoundBlaster 16 quantization errors.

LENGTH = (SAMPLES + 1)/2 - 1

SEE ALSO

048h   Set DMA Block Size

�����������������������������������������������������

0D0h   Halt DMA Operation, 8-bit

045h   Continue Auto-Initialize DMA Operation, 8-bit

0DAh   Exit Auto-Initialize DMA Operation, 8-bit

�����������������������������������������������������

090h   Auto-Initialize DMA DAC, 8-bit (High Speed)

0Cxh   Generic DAC/ADC DMA, 8-bit

------------------------------------------------------------------------------

01Fh       Auto-Initialize DMA DAC, 2-bit ADPCM Reference          SB2.0

COMMAND

DESCRIPTION

Initiates auto-initialize 2-bit ADPCM DMA transfer with reference byte.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Set DMA Block Size command (048h)

d) Perform Enable Speaker command (0D1h)

e) Setup DMA controller (mode = 058h + channel)

f) Perform Auto-Initialize DMA DAC, 2-bit ADPCM command (01Fh)

g) IRQ: Prepare next half of buffer (not always in handler)

h) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

i) Loop to G until complete

j) Perform Disable Speaker command (0D3h)

k) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

l) Perform Exit Auto-Initialize DMA Operation, 8-bit command (0DAh)

m) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

NOTES

� Exit auto-initialized mode by programming single-cycle DMA output or

with Exit Auto-Initialize DMA Operation, 8-bit (0DAh).

� Ensoniq Soundscape does not support ADPCM.

SAMPLEBYTES = (SAMPLES     + 3)/4 + 1

LENGTH      = (SAMPLEBYTES + 1)/2 - 1

SEE ALSO

048h   Set DMA Block Size

�����������������������������������������������������

0D0h   Halt DMA Operation, 8-bit

045h   Continue Auto-Initialize DMA Operation, 8-bit

0DAh   Exit Auto-Initialize DMA Operation, 8-bit

�����������������������������������������������������

017h   DMA DAC, 2-bit ADPCM Reference

�����������������������������������������������������

07Dh   Auto-Initialize DMA DAC, 4-bit ADPCM Reference

07Fh   Auto-Initialize DMA DAC, 2.6-bit ADPCM Reference

------------------------------------------------------------------------------

020h       Direct ADC, 8-bit                                       SB

COMMAND <-SAMPLEBYTE

DESCRIPTION

Inputs single sample.

PROCEDURE

a) Send Direct ADC, 8-bit command (020h)

b) Read sample

c) Wait for correct timing

NOTES

� Direct mode maximum sample rate is 23KHz.

SEE ALSO

010h   Direct DAC, 8-bit

------------------------------------------------------------------------------

024h       DMA ADC, 8-bit                                          SB

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Initiates 8-bit DMA transfer (record).

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Enable Speaker command (0D1h)

d) Setup DMA controller (mode = 044h + channel)

e) Perform DMA ADC, 8-bit command (024h)

f) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

g) Perform Disable Speaker command (0D3h)

LENGTH = SAMPLES - 1

NOTES

� SoundBlasters prior to SB16 return the first sample in direct mode.

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

0D4h   Continue DMA Operation, 8-bit

�����������������������������������������������������

02Ch   Auto-Initialize DMA ADC, 8-bit

098h   Auto-Initialize DMA ADC, 8-bit (High Speed)

0Cxh   Generic DAC/ADC DMA, 8-bit

------------------------------------------------------------------------------

02Ch           Auto-Initialize DMA ADC, 8-bit                      SB2.0

COMMAND

DESCRIPTION

Initiates auto-initialize 8-bit DMA transfer (record).

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Set DMA Block Size command (048h)

d) Perform Enable Speaker command (0D1h)

e) Setup DMA controller (mode = 054h + channel)

f) Perform Auto-Initialize DMA ADC, 8-bit command (02Ch)

g) IRQ: Prepare next half of buffer (not always in handler)

h) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

i) Loop to G until complete

j) Perform Disable Speaker command (0D3h)

k) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

l) Perform Exit Auto-Initialize DMA Operation, 8-bit command (0DAh)

m) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

NOTES

� Supports up to 23KHz (11.5KHz stereo???)

� Exit auto-initialized mode by programming single-cycle DMA output or

with Exit Auto-Initialize DMA Operation, 8-bit (0DAh).

� Use command 0Cxh to avoid SoundBlaster 16 quantization errors.

LENGTH = (SAMPLES + 1)/2 - 1

SEE ALSO

048h   Set DMA Block Size

�����������������������������������������������������

0D0h   Halt DMA Operation, 8-bit

045h   Continue Auto-Initialize DMA Operation, 8-bit

0DAh   Exit Auto-Initialize DMA Operation, 8-bit

�����������������������������������������������������

098h   Auto-Initialize DMA ADC, 8-bit (High Speed)

0Cxh   Generic DAC/ADC DMA, 8-bit

------------------------------------------------------------------------------

030h       MIDI Read Poll                                          SB

COMMAND <-MIDICODE

DESCRIPTION

Reads MIDI code.

PROCEDURE

a) Send MIDI Read Poll command (030h)

b) Read MIDI code when available

NOTES

� SoundBlasters buffer up to 64bytes of MIDI data.

SEE ALSO

031h   MIDI Read Interrupt

�����������������������������������������������������

032h   MIDI Read Timestamp Poll

033h   MIDI Read Timestamp Interrupt

�����������������������������������������������������

034h   MIDI Read Poll + Write Poll (UART)

035h   MIDI Read Interrupt + Write Poll (UART)

037h   MIDI Read Timestamp Interrupt + Write Poll (UART)

�����������������������������������������������������

038h   MIDI Write Poll

------------------------------------------------------------------------------

031h       MIDI Read Interrupt                                     SB

COMMAND {<-MIDICODE}

DESCRIPTION

Generates interrupt when MIDI code is available.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform MIDI Read Interrupt command (031h)

c) IRQ: Read MIDI code

d) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

e) Perform MIDI Read Interrupt command (031h) to terminate

NOTES

� SoundBlasters buffer up to 64bytes of MIDI data.

SEE ALSO

033h   MIDI Read Timestamp Interrupt

�����������������������������������������������������

035h   MIDI Read Interrupt + Write Poll (UART)

037h   MIDI Read Timestamp Interrupt + Write Poll (UART)

�����������������������������������������������������

030h   MIDI Read Poll

------------------------------------------------------------------------------

034h       MIDI Read Poll + Write Poll (UART)                      SB2.0

COMMAND [->MIDICODE] [<-MIDICODE]

DESCRIPTION

Enables UART mode, where all DSP read/writes are interpreted as MIDI codes.

PROCEDURE

a) Send MIDI Read Poll + Write Poll (UART) command (034h)

b) Send and receive MIDI codes

c) Reset DSP to terminate

NOTES

� Disable UART mode with DSP reset (port 02x06h)

� Read/write status must still be checked (port 02x0Eh/02x0Ch)

� SoundBlasters buffer up to 64bytes of MIDI data.

SEE ALSO

030h   MIDI Read Poll

038h   MIDI Write Poll

�����������������������������������������������������

035h   MIDI Read Interrupt + Write Poll (UART)

037h   MIDI Read Timestamp Interrupt + Write Poll (UART)

------------------------------------------------------------------------------

038h       MIDI Write Poll                                         SB

COMMAND->MIDICODE

DESCRIPTION

Writes MIDI code.

PROCEDURE

a) Send MIDI Write Poll command (038h) and data

SEE ALSO

034h   MIDI Read Poll + Write Poll (UART)

035h   MIDI Read Interrupt + Write Poll (UART)

037h   MIDI Read Timestamp Interrupt + Write Poll (UART)

�����������������������������������������������������

030h   MIDI Read Poll

------------------------------------------------------------------------------

040h       Set Time Constant                                       SB

COMMAND->TIMEBYTE

DESCRIPTION

Sets sample rate through internal I/O transfer timer.

FORMULA

TimeConstant = 256 - (1000000 / (SampleChannels * SampleRate))

NOTES

� For non-SoundBlaster Pro cards, always set SampleChannels equal to one.

SEE ALSO

041h   Set Sample Rate

------------------------------------------------------------------------------

048h       Set DMA Block Size                                      SB2.0

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Set DMA transfer size for auto-initialize and high speed modes.

SAMPLEBYTES =  SAMPLES                    8-bit

(SAMPLES-1   + 1)/2 + 1     4-bit ADPCM + Reference

(SAMPLES-1   + 2)/3 + 1   2.6-bit ADPCM + Reference

(SAMPLES-1   + 3)/4 + 1     2-bit ADPCM + Reference

LENGTH      =  SAMPLEBYTES        - 1  (Single Cycle)

(SAMPLEBYTES + 1)/2 - 1  (Auto-Init   )

NOTES

� Length is always equal to the number of bytes to transfer minus one.

SEE ALSO

01Ch   Auto-Initialize DMA DAC,   8-bit

01Fh   Auto-Initialize DMA DAC,   2-bit ADPCM

02Ch   Auto-Initialize DMA ADC,   8-bit

07Dh   Auto-Initialize DMA DAC,   4-bit ADPCM

07Fh   Auto-Initialize DMA DAC, 2.6-bit ADPCM

090h   Auto-Initialize DMA DAC,   8-bit (High Speed)

098h   Auto-Initialize DMA ADC,   8-bit (High Speed)

------------------------------------------------------------------------------

074h       DMA DAC, 4-bit ADPCM                                    SB

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Initiates 4-bit ADPCM DMA transfer with accumulated reference byte.

This operation uses 8-bit DMA mode.

PROCEDURE (after command 075h)

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Enable Speaker command (0D1h)

d) Setup DMA controller (mode = 048h + channel)

e) Perform DMA DAC, 4-bit ADPCM command (074h)

f) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

g) Perform Disable Speaker command (0D3h)

LENGTH      = (SAMPLES-1   + 1)/2

NOTES

� Supports up to 12KHz on SoundBlaster 1.x

� Ensoniq Soundscape does not support ADPCM.

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

0D4h   Continue DMA Operation, 8-bit

�����������������������������������������������������

074h   DMA DAC, 4-bit ADPCM Reference

�����������������������������������������������������

016h   DMA DAC, 2-bit ADPCM

076h   DMA DAC, 2.6-bit ADPCM

------------------------------------------------------------------------------

075h       DMA DAC, 4-bit ADPCM Reference                          SB

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Initiates 4-bit ADPCM DMA transfer with new reference byte.  This

operation uses 8-bit DMA mode.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Enable Speaker command (0D1h)

d) Setup DMA controller (mode = 048h + channel)

e) Perform DMA DAC, 4-bit ADPCM Reference command (075h)

f) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

g) Perform Disable Speaker command (0D3h)

LENGTH      = (SAMPLES-1   + 1)/2 + 1

NOTES

� Supports up to 12KHz on SoundBlaster 1.x

� Ensoniq Soundscape does not support ADPCM.

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

0D4h   Continue DMA Operation, 8-bit

�����������������������������������������������������

075h   DMA DAC, 4-bit ADPCM

�����������������������������������������������������

017h   DMA DAC, 2-bit ADPCM Reference

077h   DMA DAC, 2.6-bit ADPCM Reference

------------------------------------------------------------------------------

076h       DMA DAC, 2.6-bit ADPCM                                  SB

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Initiates 2.6-bit ADPCM DMA transfer with accumulated reference byte.

This operation uses 8-bit DMA mode.

PROCEDURE (after command 077h)

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Enable Speaker command (0D1h)

d) Setup DMA controller (mode = 048h + channel)

e) Perform DMA DAC, 2.6-bit ADPCM command (076h)

f) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

g) Perform Disable Speaker command (0D3h)

LENGTH      = (SAMPLES-1   + 2)/3

NOTES

� Supports up to 13KHz on SoundBlaster 1.x

� Ensoniq Soundscape does not support ADPCM.

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

0D4h   Continue DMA Operation, 8-bit

�����������������������������������������������������

077h   DMA DAC, 2.6-bit ADPCM Reference

�����������������������������������������������������

016h   DMA DAC, 2-bit ADPCM

074h   DMA DAC, 4-bit ADPCM

------------------------------------------------------------------------------

077h       DMA DAC, 2.6-bit ADPCM Reference                        SB

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Initiates 2.6-bit ADPCM DMA transfer with new reference byte.  This

operation uses 8-bit DMA mode.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Enable Speaker command (0D1h)

d) Setup DMA controller (mode = 048h + channel)

e) Perform DMA DAC, 2.6-bit ADPCM Reference command (077h)

f) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

g) Perform Disable Speaker command (0D3h)

LENGTH      = (SAMPLES-1   + 2)/3 + 1

NOTES

� Supports up to 13KHz on SoundBlaster 1.x

� Ensoniq Soundscape does not support ADPCM.

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

0D4h   Continue DMA Operation, 8-bit

�����������������������������������������������������

076h   DMA DAC, 2.6-bit ADPCM

�����������������������������������������������������

017h   DMA DAC, 2-bit ADPCM Reference

075h   DMA DAC, 4-bit ADPCM Reference

------------------------------------------------------------------------------

07Dh       Auto-Initialize DMA DAC, 4-bit ADPCM Reference          SB2.0

COMMAND

DESCRIPTION

Initiates auto-initialize 4-bit ADPCM DMA transfer with reference byte.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Set DMA Block Size command (048h)

d) Perform Enable Speaker command (0D1h)

e) Setup DMA controller (mode = 058h + channel)

f) Perform Auto-Initialize DMA DAC, 4-bit ADPCM command (07Dh)

g) IRQ: Prepare next half of buffer (not always in handler)

h) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

i) Loop to G until complete

j) Perform Disable Speaker command (0D3h)

k) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

l) Perform Exit Auto-Initialize DMA Operation, 8-bit command (0DAh)

m) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

NOTES

� Exit auto-initialized mode by programming single-cycle DMA output or

with Exit Auto-Initialize DMA Operation, 8-bit (0DAh).

� Ensoniq Soundscape does not support ADPCM.

SAMPLEBYTES = (SAMPLES     + 1)/2 + 1

LENGTH      = (SAMPLEBYTES + 1)/2 - 1

SEE ALSO

048h   Set DMA Block Size

�����������������������������������������������������

0D0h   Halt DMA Operation, 8-bit

045h   Continue Auto-Initialize DMA Operation, 8-bit

0DAh   Exit Auto-Initialize DMA Operation, 8-bit

�����������������������������������������������������

075h   DMA DAC, 4-bit ADPCM Reference

�����������������������������������������������������

01Fh   Auto-Initialize DMA DAC, 2-bit ADPCM Reference

07Fh   Auto-Initialize DMA DAC, 2.6-bit ADPCM Reference

------------------------------------------------------------------------------

07Fh       Auto-Initialize DMA DAC, 2.6-bit ADPCM Reference        SB2.0

COMMAND

DESCRIPTION

Initiates auto-initialize 2.6-bit ADPCM DMA transfer with reference byte.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Set DMA Block Size command (048h)

d) Perform Enable Speaker command (0D1h)

e) Setup DMA controller (mode = 058h + channel)

f) Perform Auto-Initialize DMA DAC, 2.6-bit ADPCM command (07Fh)

g) IRQ: Prepare next half of buffer (not always in handler)

h) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

i) Loop to G until complete

j) Perform Disable Speaker command (0D3h)

k) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

l) Perform Exit Auto-Initialize DMA Operation, 8-bit command (0DAh)

m) Perform Halt DMA Operation, 8-bit command (0D0h - for virtual speaker)

NOTES

� Exit auto-initialized mode by programming single-cycle DMA output or

with Exit Auto-Initialize DMA Operation, 8-bit (0DAh).

� Ensoniq Soundscape does not support ADPCM.

SAMPLEBYTES = (SAMPLES     + 2)/3 + 1

LENGTH      = (SAMPLEBYTES + 1)/2 - 1

SEE ALSO

048h   Set DMA Block Size

�����������������������������������������������������

0D0h   Halt DMA Operation, 8-bit

045h   Continue Auto-Initialize DMA Operation, 8-bit

0DAh   Exit Auto-Initialize DMA Operation, 8-bit

�����������������������������������������������������

077h   DMA DAC, 2.6-bit ADPCM Reference

�����������������������������������������������������

01Fh   Auto-Initialize DMA DAC, 2-bit ADPCM Reference

07Dh   Auto-Initialize DMA DAC, 4-bit ADPCM Reference

------------------------------------------------------------------------------

080h       Silence DAC                                             SB

COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

DESCRIPTION

Outputs block of silent samples at current sample rate, and signals

IRQ when complete.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform Set Time Constant command (040h), or otherwise set sample rate

c) Perform Silence DAC command (080h)

d) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

e) Remove IRQ handler (restore vector, restore PIC mask)

LENGTH = SAMPLES - 1

SEE ALSO

040h   Set Time Constant

------------------------------------------------------------------------------

0D0h       Halt DMA Operation, 8-bit                               SB

COMMAND

DESCRIPTION

Pauses 8-bit single-cycle DMA operation.

NOTES

� Halts every 8-bit DMA mode on SoundBlaster 16.

� BUG: Halts 16-bit DMA operation on early SoundBlaster 16s (4.04).

SEE ALSO

0D4h   Continue DMA Operation, 8-bit

0DAh   Exit Auto-Initialize DMA Operation, 8-bit

------------------------------------------------------------------------------

0D1h       Enable Speaker                                          SB

COMMAND

DESCRIPTION

Enables speaker output.

NOTES

� Reset of DSP disables speaker.

� BUG: Halts pending DMA operations on SoundBlaster 1.x.

� BUG: Enabled speaker generates sampling noise on SoundBlaster 2.0.

� BUG: Speaker MUST be disabled before sampling on SoundBlaster Pro.

� BUG: Speaker state does NOT physically change on SoundBlaster 16.

� Originator of noisy 'pops' on some models (capacitor discharge).

SEE ALSO

0D3h   Disable Speaker

------------------------------------------------------------------------------

0D3h       Disable Speaker                                         SB

COMMAND

DESCRIPTION

Disables speaker output.

NOTES

� Reset of DSP disables speaker.

� BUG: Halts pending DMA operations on SoundBlaster 1.x.

� BUG: Enabled speaker generates sampling noise on SoundBlaster 2.0.

� BUG: Speaker MUST be disabled before sampling on SoundBlaster Pro.

� BUG: Speaker state does NOT physically change on SoundBlaster 16.

� Originator of noisy 'pops' on some models (capacitor discharge).

SEE ALSO

0D1h   Enable Speaker

------------------------------------------------------------------------------

0D4h       Continue DMA Operation, 8-bit                           SB

COMMAND

DESCRIPTION

Continues a halted 8-bit single-cycle DMA operation.

NOTES

� Continues every 8-bit DMA mode on SoundBlaster 16.

� BUG: Continues 16-bit DMA operation on early SoundBlaster 16 (4.04).

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

0DAh   Exit Auto-Initialize DMA Operation, 8-bit

------------------------------------------------------------------------------

0D8h       Speaker Status                                          SB

COMMAND <-STATUS

DESCRIPTION

Determines current status of speaker.

STATUS BYTE

000h = Disabled

0FFh = Enabled

NOTES

� Reset of DSP disables speaker.

SEE ALSO

0D1h   Enable Speaker

0D3h   Disable Speaker

�����������������������������������������������������

004h   DSP Status (Obsolete)

0F1h   DSP Auxiliary Status (Obsolete)

�����������������������������������������������������

0FBh   DSP Status

0FCh   DSP Auxiliary Status

0FDh   DSP Command Status

------------------------------------------------------------------------------

0DAh       Exit Auto-Initialize DMA Operation, 8-bit               SB2.0

COMMAND

DESCRIPTION

Terminates auto-initialized 8-bit DMA operations after current block.

PROCEDURE

a) Perform Disable Speaker command (0D3h)

b) Perform Halt DMA Operation, 8-bit command (0D0h - virtual speaker)

c) Perform Exit Auto-Initialize DMA Operation, 8-bit command (0DAh)

d) Perform Halt DMA Operation, 8-bit command (0D0h - virtual speaker)

NOTES

� Halt commands are required for immediate termination and to quiet SB16.

SEE ALSO

0D0h   Halt DMA Operation, 8-bit

045h   Continue Auto-Initialize DMA, 8-bit

�����������������������������������������������������

0D3h   Disable Speaker

------------------------------------------------------------------------------

0E0h       DSP Identification                                      SB2.0

COMMAND->DATA <-NOT(DATA)

DESCRIPTION

Returns bitwise NOT of data byte.

NOTES

� Results reliable only after DSP reset on early models.

SEE ALSO

0E1h   DSP Version

0E3h   DSP Copyright

------------------------------------------------------------------------------

0E1h       DSP Version                                             SB

COMMAND <-MAJORVERSIONBYTE<-MINORVERSIONBYTE

DESCRIPTION

Determines DSP major and minor version.

MODEL                      VERSION

SoundBlaster 1.0           1.??  (1.05???)

SoundBlaster 1.5           1.??  (1.05???)

SoundBlaster 2.0           2.xx  (2.01)

SoundBlaster Pro           3.00  (???)

SoundBlaster Pro 2         3.01+ (3.01, 3.02)

SoundBlaster 16            4.0x  (4.04, 4.05)

SoundBlaster 16 SCSI-2     4.11  (4.11)

SoundBlaster AWE32         4.12+ (4.12)

NOTES

� Ensure that no other card is mapped at the same port address by

performing this command twice, checking for a consistent result.

� BUG: Some SB1.x clones errantly return version 2.00.

� BUG: Some SB16 SCSI-2s experience daughterboard communication errors.

SEE ALSO

0E0h   DSP Identification

0E3h   DSP Copyright

------------------------------------------------------------------------------

0E4h       Write Test Register                                     SB2.0

COMMAND->TESTBYTE

DESCRIPTION

Writes diagnostic register.

NOTES

� DSP reset does not clear the test register.

SEE ALSO

0E8h   Read Test Register

------------------------------------------------------------------------------

0E8h       Read Test Register                                      SB2.0

COMMAND <-TESTBYTE

DESCRIPTION

Reads diagnostic register.

NOTES

� DSP reset does not clear the test register.

SEE ALSO

0E4h   Write Test Register

------------------------------------------------------------------------------

0F0h       Sine Generator                                          SB

COMMAND

DESCRIPTION

Diagnostic peak-to-peak sine wave generator.

NOTES

� Disable sound with DSP reset.

� Sinusodal frequency is about 2KHz.

� Sets time constant to approximately 0C0h (15625Hz).

� Enables speaker on models prior to SoundBlaster 16.

------------------------------------------------------------------------------

0F2h       IRQ Request, 8-bit                                      SB

COMMAND

DESCRIPTION

Triggers 8-bit interrupt.

PROCEDURE

a) Install IRQ handler (hook vector, update PIC mask)

b) Perform IRQ Request, 8-bit command (0F2h)

c) IRQ: Acknowledge IRQ (input from IRQ Acknowledge, 8-bit port - 02x0Eh;

perform Generic EOI (020h) to appropriate PICs)

SEE ALSO

0F3h   Interrupt Request, 16-bit

==============================================================================

MIXER REGISTERS

------------------------------------------------------------------------------

000h       Reset                                       Write       SBPro

001h       Status                                      Read        SBPro

004h       DAC Level                                   Read/Write  SBPro

00Ah       Microphone Level                            Read/Write  SBPro

022h       Master Volume                               Read/Write  SBPro

026h       FM Level                                    Read/Write  SBPro

028h       CD Audio Level                              Read/Write  SBPro

02Eh       Line In Level                               Read/Write  SBPro

030h       Master Volume Left                          Read/Write  SB16

031h       Master Volume Right                         Read/Write  SB16

032h       DAC Level Left                              Read/Write  SB16

033h       DAC Level Right                             Read/Write  SB16

034h       FM Level Left                               Read/Write  SB16

035h       FM Level Right                              Read/Write  SB16

036h       CD Audio Level Left                         Read/Write  SB16

037h       CD Audio Level Right                        Read/Write  SB16

038h       Line In Level Left                          Read/Write  SB16

039h       Line In Level Right                         Read/Write  SB16

03Ah       Microphone Level                            Read/Write  SB16

03Bh       PC Speaker Level                            Read/Write  SB16

03Ch       Output Control                              Read/Write  SB16

03Dh       Input Control Left                          Read/Write  SB16

03Eh       Input Control Right                         Read/Write  SB16

03Fh       Input Gain Control Left                     Read/Write  SB16

040h       Input Gain Control Right                    Read/Write  SB16

041h       Output Gain Control Left                    Read/Write  SB16

042h       Output Gain Control Right                   Read/Write  SB16

043h       Automatic Gain Control (AGC)                Read/Write  SB16

044h       Treble Left                                 Read/Write  SB16

045h       Treble Right                                Read/Write  SB16

046h       Bass Left                                   Read/Write  SB16

047h       Bass Right                                  Read/Write  SB16

080h       IRQ Select                                  Read/Write  SB16

081h       DMA Select                                  Read/Write  SB16

082h       IRQ Status                                  Read        SB16

------------------------------------------------------------------------------

000h       Reset                                       Write       SBPro

DESCRIPTION

Resets mixer to default settings.

PROCEDURE

a) Load index register

b) Wait 100�s

c) Write reset stop

RESET BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reset Stop (0)

�   �   �   �   �   �   �������� Reserved   (0)

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

NOTES

� Defaults documented for each register (SBPro, SB16)

------------------------------------------------------------------------------

001h       Status                                      Read        SBPro

DESCRIPTION

Contains previously selected register value.

STATUS BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Mixer Data Register value

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

NOTES

� SoundBlaster 16 sets bit 7 if previous mixer index invalid.

� Status bytes initially 080h on startup for all but level bytes (SB16)

------------------------------------------------------------------------------

004h       DAC Level                                   Read/Write  SBPro

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Right DAC Level bit  0  (SB16, SBPro=1)

�   �   �   �   �   �   �������� Right DAC Level bit 3-1 (SBPro)

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Left  DAC Level bit  0  (SB16, SBPro=1)

�   �   ������������������������ Left  DAC Level bit 3-1 (SBPro)

�   �������������������������Ĵ

�������������������������������

DEFAULT

099h, 0CCh

NOTES

� Mixer detection can be accomplished by looking for this register index.

� SoundBlaster 16 will automatically update registers 031h and 032h,

transferring bits 7-4 and setting bit 3.

SEE ALSO

032h   DAC Level Left                      Read/Write

033h   DAC Level Right                     Read/Write

------------------------------------------------------------------------------

00Ah       Microphone Level                            Read/Write  SBPro

LEVEL BYTE

�������������������������������ͻ

�[7]�[6]�[5]�[4]�[3]� 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Microphone Level bit  0  (SB16, SBPro=0)

�   �   �   �   �   �   �������� Microphone Level bit 2-1 (SBPro)

�   �   �   �   �   �����������

�   �   �   �   ���������������� Reserved (0)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

011h, 000h

SEE ALSO

03Ah   Microphone Level                    Read/Write

------------------------------------------------------------------------------

022h       Master Volume                               Read/Write  SBPro

VOLUME BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Right Master bit  0  (SB16, SBPro=1)

�   �   �   �   �   �   �������� Right Master bit 3-1 (SBPro)

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Left  Master bit  0  (SB16, SBPro=1)

�   �   ������������������������ Left  Master bit 3-1 (SBPro)

�   �������������������������Ĵ

�������������������������������

DEFAULT

011h, 0CCh

NOTES

� SoundBlaster 16 will automatically update registers 030h and 031h,

transferring bits 7-4 and setting bit 3.

� SoundBlaster Pro register 002h is the same as this.

SEE ALSO

002h   Master Volume                       Read/Write

�����������������������������������������������������

030h   Master Volume Left                  Read/Write

031h   Master Volume Right                 Read/Write

------------------------------------------------------------------------------

026h       FM Level                                    Read/Write  SBPro

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Right FM Level bit  0  (SB16, SBPro=1)

�   �   �   �   �   �   �������� Right FM Level bit 3-1 (SBPro)

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Left  FM Level bit  0  (SB16, SBPro=1)

�   �   ������������������������ Left  FM Level bit 3-1 (SBPro)

�   �������������������������Ĵ

�������������������������������

DEFAULT

011h, 0CCh

NOTES

� SoundBlaster 16 will automatically update registers 033h and 034h,

transferring bits 7-4 and setting bit 3.

� SoundBlaster Pro will automatically update register 006h,

transferring bits 3-1 and setting bit 0.

SEE ALSO

006h   FM Output Control                   Read/Write

�����������������������������������������������������

034h   FM Level Left                       Read/Write

035h   FM Level Right                      Read/Write

------------------------------------------------------------------------------

028h       CD Audio Level                              Read/Write  SBPro

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Right CD Audio bit  0  (SB16, SBPro=1)

�   �   �   �   �   �   �������� Right CD Audio bit 3-1 (SBPro)

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Left  CD Audio bit  0  (SB16, SBPro=1)

�   �   ������������������������ Left  CD Audio bit 3-1 (SBPro)

�   �������������������������Ĵ

�������������������������������

DEFAULT

011h, 000h

NOTES

� SoundBlaster 16 will automatically update registers 035h and 036h,

transferring bits 7-4 and setting bit 3.

SEE ALSO

036h   CD Audio Level Left                 Read/Write

037h   CD Audio Level Right                Read/Write

------------------------------------------------------------------------------

02Eh       Line In Level                               Read/Write  SBPro

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Right Line In bit  0  (SB16, SBPro=1)

�   �   �   �   �   �   �������� Right Line In bit 3-1 (SBPro)

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Left  Line In bit  0  (SB16, SBPro=1)

�   �   ������������������������ Left  Line In bit 3-1 (SBPro)

�   �������������������������Ĵ

�������������������������������

DEFAULT

011h, 000h

NOTES

� SoundBlaster 16 will automatically update registers 037h and 038h,

transferring bits 7-4 and setting bit 3.

SEE ALSO

038h   Line In Level Left                  Read/Write

039h   Line In Level Right                 Read/Write

------------------------------------------------------------------------------

030h       Master Volume Left                          Read/Write  SB16

VOLUME BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Left Master Volume (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 0C0h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 7-4

of register 022h

� SoundBlaster Pro selects register 020h for 030h.

SEE ALSO

031h   Master Volume Right                 Read/Write

�����������������������������������������������������

022h   Master Volume                       Read/Write

------------------------------------------------------------------------------

031h       Master Volume Right                         Read/Write  SB16

VOLUME BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Right Master Volume (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 0C0h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 3-0

of register 022h.

SEE ALSO

030h   Master Volume Left                  Read/Write

�����������������������������������������������������

022h   Master Volume                       Read/Write

------------------------------------------------------------------------------

032h       DAC Level Left                              Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Left DAC Level (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 0C0h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 7-4

of register 024h.

� SoundBlaster Pro selects register 022h for 032h.

SEE ALSO

033h   DAC Level Right                     Read/Write

�����������������������������������������������������

004h   DAC Level                           Read/Write

------------------------------------------------------------------------------

033h       DAC Level Right                             Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Right DAC Level (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 0C0h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 3-0

of register 024h.

SEE ALSO

032h   DAC Level Left                      Read/Write

�����������������������������������������������������

004h   DAC Level                           Read/Write

------------------------------------------------------------------------------

034h       FM Level Left                               Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Left FM Level (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 0C0h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 7-4

of register 026h.

SEE ALSO

035h   FM Level Right                      Read/Write

�����������������������������������������������������

006h   FM Output Control                   Read/Write

026h   FM Level                            Read/Write

------------------------------------------------------------------------------

035h       FM Level Right                              Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Right FM Level (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 0C0h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 3-0

of register 026h.

SEE ALSO

034h   FM Level Left                       Read/Write

�����������������������������������������������������

006h   FM Output Control                   Read/Write

026h   FM Level                            Read/Write

------------------------------------------------------------------------------

036h       CD Audio Level Left                         Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Left CD Audio Level (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 000h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 7-4

of register 028h.

� SoundBlaster Pro selects register 026h for 036h.

SEE ALSO

037h   CD Audio Level Right                Read/Write

�����������������������������������������������������

028h   CD Audio Level                      Read/Write

------------------------------------------------------------------------------

037h       CD Audio Level Right                        Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Right CD Audio Level (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 000h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 3-0

of register 028h.

SEE ALSO

036h   CD Audio Level Left                 Read/Write

�����������������������������������������������������

028h   CD Audio Level                      Read/Write

------------------------------------------------------------------------------

038h       Line In Level Left                          Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Left Line In Level (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 000h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 7-4

of register 02Eh.

� SoundBlaster Pro selects register 028h for 038h.

SEE ALSO

039h   Line In Level Right                 Read/Write

�����������������������������������������������������

02Eh   Line In Level                       Read/Write

------------------------------------------------------------------------------

039h       Line In Level Right                         Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Right Line In Level (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 000h

NOTES

� SoundBlaster 16 will automatically transfer bits 7-4 to bits 3-0

of register 02Eh.

SEE ALSO

038h   Line In Level Left                  Read/Write

�����������������������������������������������������

02Eh   Line In Level                       Read/Write

------------------------------------------------------------------------------

03Ah       Microphone Level                            Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 � 3 �[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   �����������

�   �   �   �   ���������������� Microphone Level (2dB)

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 000h

SEE ALSO

00Ah   Microphone Level                    Read/Write

------------------------------------------------------------------------------

03Bh       PC Speaker Level                            Read/Write  SB16

LEVEL BYTE

�������������������������������ͻ

� 7 � 6 �[5]�[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   �����������������������

�   ���������������������������� PC Speaker Level (2dB)

�������������������������������

DEFAULT

N/A, 000h

------------------------------------------------------------------------------

03Ch       Output Control                              Read/Write  SB16

CONTROL BYTE

�������������������������������ͻ

�[7]�[6]�[5]� 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Microphone     (0 = Off, 1 = On)

�   �   �   �   �   �   �������� Right CD Audio (0 = Off, 1 = On)

�   �   �   �   �   ������������ Left  CD Audio (0 = Off, 1 = On)

�   �   �   �   ���������������� Right Line In  (0 = Off, 1 = On)

�   �   �   �������������������� Left  Line In  (0 = Off, 1 = On)

�   �   ������������������������ Reserved (0)

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 01Fh

SEE ALSO

041h   Output Gain Control Left            Read/Write

042h   Output Gain Control Right           Read/Write

�����������������������������������������������������

036h   CD Audio Level Left                 Read/Write

037h   CD Audio Level Right                Read/Write

038h   Line In Level Left                  Read/Write

039h   Line In Level Right                 Read/Write

03Ah   Microphone Level                    Read/Write

�����������������������������������������������������

00Eh   Output/Stereo Select                Read/Write

------------------------------------------------------------------------------

03Dh       Input Control Left                          Read/Write  SB16

CONTROL BYTE

�������������������������������ͻ

�[7]� 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Microphone     (0 = Off, 1 = On)

�   �   �   �   �   �   �������� Right CD Audio (0 = Off, 1 = On)

�   �   �   �   �   ������������ Left  CD Audio (0 = Off, 1 = On)

�   �   �   �   ���������������� Right Line In  (0 = Off, 1 = On)

�   �   �   �������������������� Left  Line In  (0 = Off, 1 = On)

�   �   ������������������������ Right FM       (0 = Off, 1 = On)

�   ���������������������������� Left  FM       (0 = Off, 1 = On)

�������������������������������� Reserved (0)

DEFAULT

N/A, 015h

NOTES

� Register controls remapping of left input channel

(off, left, right, or left and right)

SEE ALSO

03Eh   Input Control Right                 Read/Write

�����������������������������������������������������

03Fh   Input Gain Control Left             Read/Write

�����������������������������������������������������

034h   FM Level Left                       Read/Write

035h   FM Level Right                      Read/Write

036h   CD Audio Level Left                 Read/Write

037h   CD Audio Level Right                Read/Write

038h   Line In Level Left                  Read/Write

039h   Line In Level Right                 Read/Write

03Ah   Microphone Level                    Read/Write

�����������������������������������������������������

00Ch   Input/Filter Select                 Read/Write

------------------------------------------------------------------------------

03Eh       Input Control Right                         Read/Write  SB16

CONTROL BYTE

�������������������������������ͻ

�[7]� 6 � 5 � 4 � 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Microphone     (0 = Off, 1 = On)

�   �   �   �   �   �   �������� Right CD Audio (0 = Off, 1 = On)

�   �   �   �   �   ������������ Left  CD Audio (0 = Off, 1 = On)

�   �   �   �   ���������������� Right Line In  (0 = Off, 1 = On)

�   �   �   �������������������� Left  Line In  (0 = Off, 1 = On)

�   �   ������������������������ Right FM       (0 = Off, 1 = On)

�   ���������������������������� Left  FM       (0 = Off, 1 = On)

�������������������������������� Reserved (0)

DEFAULT

N/A, 00Bh

NOTES

� Register controls remapping of right input channel

(off, left, right, or left and right)

� SoundBlaster Pro selects register 02Eh for 03Eh.

SEE ALSO

03Dh   Input Control Left                  Read/Write

�����������������������������������������������������

040h   Input Gain Control Right            Read/Write

�����������������������������������������������������

034h   FM Level Left                       Read/Write

035h   FM Level Right                      Read/Write

036h   CD Audio Level Left                 Read/Write

037h   CD Audio Level Right                Read/Write

038h   Line In Level Left                  Read/Write

039h   Line In Level Right                 Read/Write

03Ah   Microphone Level                    Read/Write

�����������������������������������������������������

00Ch   Input/Filter Select                 Read/Write

------------------------------------------------------------------------------

03Fh       Input Gain Control Left                     Read/Write  SB16

GAIN BYTE

�������������������������������ͻ

� 7 � 6 �[5]�[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   �����������������������

�   ���������������������������� Left Input Gain Control (2^x)

�������������������������������

DEFAULT

N/A, 000h

SEE ALSO

040h   Input Gain Control Right            Read/Write

�����������������������������������������������������

043h   Automatic Gain Control (AGC)        Read/Write

�����������������������������������������������������

03Dh   Input Control Left                  Read/Write

------------------------------------------------------------------------------

040h       Input Gain Control Right                    Read/Write  SB16

GAIN BYTE

�������������������������������ͻ

� 7 � 6 �[5]�[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   �����������������������

�   ���������������������������� Right Input Gain Control (2^x)

�������������������������������

DEFAULT

N/A, 000h

SEE ALSO

03Fh   Input Gain Control Left             Read/Write

�����������������������������������������������������

043h   Automatic Gain Control (AGC)        Read/Write

�����������������������������������������������������

03Dh   Input Control Right                 Read/Write

------------------------------------------------------------------------------

041h       Output Gain Control Left                    Read/Write  SB16

GAIN BYTE

�������������������������������ͻ

� 7 � 6 �[5]�[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   �����������������������

�   ���������������������������� Left Output Gain Control (2^x)

�������������������������������

DEFAULT

N/A, 000h

NOTES

� Output Gain Control has no effect when internal amplifier disabled.

SEE ALSO

042h   Output Gain Control Right           Read/Write

�����������������������������������������������������

03Ch   Output Control                      Read/Write

------------------------------------------------------------------------------

042h       Output Gain Control Right                   Read/Write  SB16

GAIN BYTE

�������������������������������ͻ

� 7 � 6 �[5]�[4]�[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   �����������������������

�   ���������������������������� Right Output Gain Control (2^x)

�������������������������������

DEFAULT

N/A, 000h

NOTES

� Output Gain Control has no effect when internal amplifier disabled.

SEE ALSO

041h   Output Gain Control Left            Read/Write

�����������������������������������������������������

03Ch   Output Control                      Read/Write

------------------------------------------------------------------------------

043h       Automatic Gain Control (AGC)                Read/Write  SB16

AGC BYTE

�������������������������������ͻ

�[7]�[6]�[5]�[4]�[3]�[2]�[1]� 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Automatic Gain Control (1 = Enable)

�   �   �   �   �   �   �������� Reserved (0)

�   �   �   �   �   ���������Ĵ

�   �   �   �   �������������Ĵ

�   �   �   �����������������Ĵ

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 000h

NOTES

� Automatic Gain Control automatically maximizes input signal strength.

SEE ALSO

03Fh   Input Gain Control Left             Read/Write

040h   Input Gain Control Right            Read/Write

------------------------------------------------------------------------------

044h       Treble Left                                 Read/Write  SB16

TREBLE BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 �[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Treble Left (2dB, -14dB to 14dB)

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 080h

SEE ALSO

045h   Treble Right                        Read/Write

�����������������������������������������������������

046h   Bass Left                           Read/Write

------------------------------------------------------------------------------

045h       Treble Right                                Read/Write  SB16

TREBLE BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 �[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Treble Right (2dB, -14dB to 14dB)

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 080h

SEE ALSO

044h   Treble Left                         Read/Write

�����������������������������������������������������

047h   Bass Right                          Read/Write

------------------------------------------------------------------------------

046h       Bass Left                                   Read/Write  SB16

BASS BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 �[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Bass Left (2dB, -14dB to 14dB)

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 080h

SEE ALSO

047h   Bass Right                          Read/Write

�����������������������������������������������������

044h   Treble Left                         Read/Write

------------------------------------------------------------------------------

047h       Bass Right                                  Read/Write  SB16

BASS BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 �[3]�[2]�[1]�[0]�

�������������������������������ͼ

�   �   �   �   �   �   �   ���� Reserved (0)

�   �   �   �   �   �   �����Ĵ

�   �   �   �   �   ���������Ĵ

�   �   �   �   ���������������

�   �   �   �������������������� Bass Right (2dB, -14dB to 14dB)

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

DEFAULT

N/A, 080h

SEE ALSO

046h   Bass Left                           Read/Write

�����������������������������������������������������

045h   Treble Right                        Read/Write

------------------------------------------------------------------------------

080h       IRQ Select                                  Read/Write  SB16

SELECT BYTE

�������������������������������ͻ

�[7]�[6]�[5]�[4]� 3 � 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� IRQ2  (0 = Disable, 1 = Enable)

�   �   �   �   �   �   �������� IRQ5  (0 = Disable, 1 = Enable)

�   �   �   �   �   ������������ IRQ7  (0 = Disable, 1 = Enable)

�   �   �   �   ���������������� IRQ10 (0 = Disable, 1 = Enable)

�   �   �   �������������������� Reserved (1)

�   �   ���������������������Ĵ

�   �������������������������Ĵ

�������������������������������

NOTES

� Unaffected by mixer reset or soft reboot.

� Enabling multiple bits enables multiple IRQs.

SEE ALSO

081h   DMA Select                          Read/Write

�����������������������������������������������������

082h   IRQ Status                          Read

------------------------------------------------------------------------------

081h       DMA Select                                  Read/Write  SB16

SELECT BYTE

�������������������������������ͻ

� 7 � 6 � 5 �[4]� 3 �[2]� 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� DMA0, 8-bit  (0 = Disable, 1 = Enable)

�   �   �   �   �   �   �������� DMA1, 8-bit  (0 = Disable, 1 = Enable)

�   �   �   �   �   ������������ DMA2, Reserved (0)

�   �   �   �   ���������������� DMA3, 8-bit  (0 = Disable, 1 = Enable)

�   �   �   �������������������� DMA4, Reserved (0)

�   �   ������������������������ DMA5, 16-bit (0 = Disable, 1 = Enable)

�   ���������������������������� DMA6, 16-bit (0 = Disable, 1 = Enable)

�������������������������������� DMA7, 16-bit (0 = Disable, 1 = Enable)

NOTES

� Unaffected by mixer reset or soft reboot.

� Enabling multiple 8 or 16-bit DMA bits enables multiple DMA channels.

� Disabling all 8-bit DMA channel bits disables 8-bit DMA requests,

including translated 16-bit DMA requests.

� Disabling all 16-bit DMA channel bits enables translation of 16-bit DMA

requests to 8-bit ones, using the selected 8-bit DMA channel.

SEE ALSO

080h   IRQ Select                          Read/Write

------------------------------------------------------------------------------

082h       IRQ Status                                  Read        SB16

STATUS BYTE

�������������������������������ͻ

� 7 � 6 � 5 � 4 �[3]� 2 � 1 � 0 �

�������������������������������ͼ

�   �   �   �   �   �   �   ���� IRQ Status, 8-bit   (1 = Active)

�   �   �   �   �   �   �������� IRQ Status, 16-bit  (1 = Active)

�   �   �   �   �   ������������ IRQ Status, MPU-401 (1 = Active)

�   �   �   �   ���������������� Reserved (0)

�   �   �   �������������������� ???

�   �   ���������������������Ĵ  (01h = v4.04, 02h = v4.05, 08h = v4.12)

�   �������������������������Ĵ

�������������������������������

SEE ALSO

080h   IRQ Select                          Read/Write

==============================================================================

GLOSSARY

------------------------------------------------------------------------------

Adaptive Differential Pulse Code Modulation (ADPCM)

Creative Labs ADPCM does not follow the CCITT ADPCM recommendations,

but does provide a similar quality compression scheme for 8-bit samples

(down to 2, 2.6, or 4-bits per sample).

Creative Labs ADPCM is not supported by many SoundBlaster clones.

�����������������������������������������������������������������������������

Advanced Signal Processor (ASP, CSP)

Creative Labs proprietary programmable signal processor.

Although development tools for the ASP are scarce, Creative Labs has

released demo software implementing QSound technology and various

compression schemes (from IMA ADPCM to �-law and �-law).

Recently renamed from ASP to CSP (Creative Signal Processor) to avoid

confusion with a U.S. company of the same name.

�����������������������������������������������������������������������������

Aliasing

Digital artifacts indicative of finite numerical representation, especially

noticeble at areas of high deviation.

�����������������������������������������������������������������������������

Amplitude

Maximum departure of the value of an alternating current or wave from an

average value.  In acoustics, amplitude is intimately related to loudness.

�����������������������������������������������������������������������������

Amplitude Modulation (AM)

Carrier wave amplitude modulation in response to the strength of a signal.

�����������������������������������������������������������������������������

Analog

Representation with periodic waves.  Contrast digital.

�����������������������������������������������������������������������������

Analog-To-Digital Convertor (ADC)

Device which converts from an analog signal (such as sound) to a digital one.

�����������������������������������������������������������������������������

Auto-Initialize Mode (AI)

Mode where DMA controller processes multiple blocks without explicit

reprogramming.  Contrast single-cycle mode.

�����������������������������������������������������������������������������

Bass

Acoustic quality describing sounds of lower frequency in the spectrum.

�����������������������������������������������������������������������������

Creative Music Synthesis??? (C/MS)

Obsolete AM synthesizer chipset implementing twelve stereo sine generators

modulated in predefined banks by one of four programmable noise generators.

Originally implemented on the Creative Labs GameBlaster card; supported by

the SoundBlaster 1.0 and 1.5 (as an upgrade option).

�����������������������������������������������������������������������������

Decibel (dB)

Unit of measurement describing the ratio of two acoustic, mechanical, or

electrical signal strengths; expressed as ten times the common logarithm

of their power ratio.

�����������������������������������������������������������������������������

Digital

Representation with finite numerical elements.  Contrast analog.

�����������������������������������������������������������������������������

Digital Signal Processor (DSP)

Device which performs mathematical operations across a digital waveform.

SoundBlasters barely qualify, with only ADPCM to their name.

�����������������������������������������������������������������������������

Digital-To-Analog Convertor (DAC)

Device which converts from a digital signal to an analog one.

�����������������������������������������������������������������������������

Direct Memory Access (DMA)

Method of using external hardware to perform block memory copy operations

so the central processing unit can be dedicated to more sophisticated tasks.

�����������������������������������������������������������������������������

Frequency

Number of repetitive occurences, or cycles, within a given time domain.

�����������������������������������������������������������������������������

Frequency Modulation (FM)

Carrier wave frequency modulation in response to a signal; resulting in

decreased distortion levels.

�����������������������������������������������������������������������������

Frequency Modulation Synthesis (FM Synthesis)

Describes a method of synthesizing sound using a programmable carrier and

one or more programmable frequency modulators.  Popularized by Fairlight.

Yamaha OPL-hardware implementation is closer to phase modulation.

�����������������������������������������������������������������������������

Gain

Ratio of increase of input over output in an amplifier.

�����������������������������������������������������������������������������

General MIDI

Standardized 128-entry patch map and associated specifications.

Patch quality differs greatly across implementations.

Regulated by the MIDI Manufacturer's Association (MMA).

�����������������������������������������������������������������������������

Hertz (Hz)

Unit of frequency measurement describing number of cycles per second.

�����������������������������������������������������������������������������

High Speed Mode

SoundBlaster DMA transfer mode operating at a frequency greater than 23kHz,

using up clocks cycles otherwise destined for DSP command interpretation.

Obsolete on SoundBlaster 16, but supported. (unofficial)

�����������������������������������������������������������������������������

Mixer

SoundBlaster component analagous to an army of volume controls.

�����������������������������������������������������������������������������

MPU-401

Developed by Roland, the MPU-401 is the industry standard MIDI interface.

It can operate either as an intelligent or a dumb UART.

�����������������������������������������������������������������������������

Music Instrument Digital Interface (MIDI)

Standard for communication between electronic musical instruments, related

hardware, and digital computers.  Serial protocol operating at 38.4Kbps.

�����������������������������������������������������������������������������

Nyquist Theorem

Accurate reproduction of a signal demands that the sampling frequency be

at least twice the rate of the highest frequency in the source signal.

�����������������������������������������������������������������������������

Pulse Code Modulation (PCM)

Digital sound reproduction method that outputs discrete amplitude values

at a steady rate to produce an analog sound.

�����������������������������������������������������������������������������

QSound (tm)

Filtering technique providing a larger panning arc than traditional

stereo techniques (180�).  Licensed by Creative Labs.

�����������������������������������������������������������������������������

Sample

Digital representation of analog signal.

�����������������������������������������������������������������������������

Single Cycle Mode (SC)

Mode where DMA controller processes a single block.  Contrast auto-init mode.

�����������������������������������������������������������������������������

Treble

Acoustic quality describing sounds of higher frequency in the spectrum.

�����������������������������������������������������������������������������

Universal Asynchronous Receiver/Transmitter (UART)

Device responsible for conversion to or from a serial bit stream.

�����������������������������������������������������������������������������

Wave Table Synthesis

Synthesis method employing sampled instruments and sounds as opposed to

artifically generated sound effects.

�����������������������������������������������������������������������������

==============================================================================

ACKNOWLEDGEMENTS

------------------------------------------------------------------------------

Ron Arts, Chris Barrett, Hussam Eassa, Brandom Hume, Cornel Huth,

Draeden/VLA, Jeffery Lee, Alain Jacques, Jerry Joplin, Eckard Lehmann,

Juan Carlos Leon, Bob Manson, Tom Marshall, Richard Mulder, Josha Munnik,

Eric Oostendorp, Tino Riethmueller, Jim Roberts, Peter Roberts,

Jens-Uwe Rumstich, Daniel Sachs, Edward Schlunder, Peter Sprenger,

Heiko Stassburg, Mark Stehr, Jim Vieira

==============================================================================

REFERENCES

------------------------------------------------------------------------------

STANDARDS

����������������������������������������������������������������������������

CCITT G.721 - ADPCM @ 32 kbits/sec

CCITT G.723 - ADPCM @ 24 and 40 kbits/sec

����������������������������������������������������������������������������

PUBLICATIONS

����������������������������������������������������������������������������

MIDI Processing Unit MPU-401 Technical Reference v1.5

Roland Corporation                                   Roland Corp.   1985

����������������������������������������������������������������������������

Schaum's Outline: Theory and Problems of Acoustics

William Seto                                         McGraw-Hill    1971

����������������������������������������������������������������������������

Yamaha OPL2 Technical Reference                       CAT#: LSI-6MF2622

Yamaha Corporation                                   Yamaha Corp.

+1 408 437-3133 (USA)

����������������������������������������������������������������������������

Yamaha OPL3 Technical Reference                       CAT#: LSI-2438120

Yamaha Corporation                                   Yamaha Corp.

+1 408 437-3133 (USA)

����������������������������������������������������������������������������

BOOKS

����������������������������������������������������������������������������

The SoundBlaster Book                                 ISBN: 3-88745-560-6

Josha Munnik, Eric Oostendorp                        SYBEX          1992

����������������������������������������������������������������������������

Das SoundBlaster Profibuch                            ISBN: 3-89319-583-1

Eckard Lehmann, Tino Riethmueller, Heiko Stassburg   Addison-Wesley 1993

����������������������������������������������������������������������������

FILES

����������������������������������������������������������������������������

Audio File Formats FAQ                                AudioFormats.*

Guido van Rossum

ftp.cwi.nl::pub/audio                               TEXT

����������������������������������������������������������������������������

DMA Programming Information                           VLA_DMA.ZIP      3K

Draeden/VLA                                         1993

x2ftp.oulu.fi::pub/msdos/programming/vla            TEXT

����������������������������������������������������������������������������

Joystick Programming Information                      GAMEPORT.INF     3K

Bill Frolik                                         1990

x2ftp.oulu.fi::pub/msdos/programming/specs          TEXT

����������������������������������������������������������������������������

MIDI Standards Documentation                          MIDI*.*

Various

ftp.cs.ruu.nl::/pub/MIDI/DOC                        TEXT

����������������������������������������������������������������������������

OPL2 Programming Information                          ADLIB_SB.TXT    20K

Jeffery Lee                                         1992

x2ftp.oulu.fi::pub/msdos/programming/faq            TEXT

����������������������������������������������������������������������������

OPL3 Programming Information                          OPL3_DOC.ARJ    18K

Vladimir Arnost                                     1994

x2ftp.oulu.fi::pub/msdos/programming/docs           TEXT, GIF

����������������������������������������������������������������������������

PC Game Programmer's Enyclopedia v1.0                 PCGPE10.ZIP    702K

Mark Feldman                                        1994

x2ftp.oulu.fi::pub/msdos/programming/gpe            TEXT, PASCAL

����������������������������������������������������������������������������

VBE/AI SDK v1.03                                      VAISDK.EXE     388K

Video Electronic Standards Association              1994

ftp.wi.leidenuniv.nl:/pub/audio/programming         TEXT, C

����������������������������������������������������������������������������

Virtual DMA Specification (VDS)                       PW0519.ZIP      47K

John Hagerson of Microsoft                          1992

ftp.microsoft.com::Softlib/MSLFILES                 TEXT, WORD

����������������������������������������������������������������������������

SOURCE

����������������������������������������������������������������������������

SoundBlaster Freedom Project v3.0                     SBF3.ZIP        85K

Jeff Bird, Christopher Box                          1994

x2ftp.oulu.fi::pub/msdos/programming/music          C, ASM

����������������������������������������������������������������������������

SoundBlaster Programming v1.0                         SBPROG10.ZIP    36K

Christopher Box                                     1993

x2ftp.oulu.fi::pub/msdos/programming/music          C++, ASM

����������������������������������������������������������������������������

SoundX SoundBlaster Library                           SOUNDX.ZIP      32K

Peter Sprenger                                      1993

x2ftp.oulu.fi::pub/msdos/programming/music          C, ASM

����������������������������������������������������������������������������

SNDBLST v4.0                                          SNDBLST4.ZIP    96K

David Welch                                         1993

x2ftp.oulu.fi::pub/msdos/programming/music          C

����������������������������������������������������������������������������

NEWSGROUPS

����������������������������������������������������������������������������

SoundBlaster Programming

alt.sb.programmer

Soundcard Technical Discussion

comp.sys.ibm.pc.soundcard.tech

comp.sys.ibm.pc.soundcard.misc

����������������������������������������������������������������������������

==============================================================================

CONTACT ADDRESSES

------------------------------------------------------------------------------

Andr� Baresel      baresel@informatik.hu-berlin.de     (Internet )

Cyder of

Green Apple       Andr� Baresel                       (Snailmail)

Hans-Grade-Ring 5

14480 Potsdam

Germany

Craig Jackson      Craig.Jackson@launchpad.unc.edu     (Internet )

Craig Jackson                       (Snailmail)

119 Penwood Drive

Cary, North Carolina 27511

United States of America